# Section 5.52.  Linear Measures

## A.  Application

A.1. - This code applies to any linear measure or measure of length, whether flexible or inflexible, permanently installed or portable.

A.2. - See also Sec. 1.10; General Code requirements.

## S.  Specifications

S.1.M.  Units. - A linear measure may be in total length, and the total length may be subdivided in any or all of the following:

(a)	centimeters and tenths of the centimeter;

(b)	meters; and

(c)	multiples of meters

A 1-meter measure may be graduated, in addition, to show 0.1 m and multiples of 0.1 m subdivisions.

S.1.  Units. - A linear measure may be in total length, and the total length may be subdivided in any or all of the following:

(a)	inches and binary submultiples of the inch;

(b)	feet;

(c)	yards and multiples of yards.

A 1-yard measure may be graduated, in addition, to show ⅓ yd and ⅔ yd subdivisions.  A flexible tape may be graduated in tenths or hundredths of a foot, or both tenths and hundredths of a foot.  (Any other subdivisions are allowable only on measures of special purposes and when required for such purposes.)

S.2.  Material.

S.2.1.  Flexible Tape. - A flexible tape shall be made of metal.

S.2.2.  End Measure. - If an end measure is made of material softer than brass, the ends of the measure shall be protected by brass (or other metal at least equally hard) securely attached.

S.3.  Finish. - Measures shall be smoothly finished.

S.4.  Design.

S.4.1.  Rigid Measure. - A rigid measure shall be straight.

S.4.2.  Folding Measure. - A folding measure shall open to a definite stop, and when so opened shall be straight.

S.5.  Graduations.

S.5.1.  General. - Graduations shall be perpendicular to the edge of the measure.

S.5.2.  Width. - The width of the graduations on any measure shall not exceed one-half the width of the smallest graduated interval on the measure, and in no case shall be wider than 0.75 mm (0.03 in).

(Amended 1982)

## T.  Tolerances

T.1.  For Measures Except Metal Tapes. - Maintenance tolerances in excess and in deficiency for measures except metal tapes shall be as shown in Table 1.  Acceptance tolerances shall be one-half the maintenance tolerances.

| Table 1. Maintenance Tolerances, in Excess and in Deficiency, for Linear Measures Except Metal Tapes |  |
| --- | --- |
| Nominal Interval from Zero | Tolerance |
| feet | inch |
| ½ or less 1 2 3 4 5 6 | 1/64 1/32 1/16 3/32 1/8 5/32 3/16 |

T.2.  For Metal Tapes. - Maintenance and acceptance tolerances in excess and in deficiency for metal tapes shall be as shown in Table 2.  Tapes of 10 m (25 ft) or over shall be tested at a tension resulting from a load of 5 kg (10 lb).  Tapes less than 10 m (25 ft) shall be tested at a tension resulting from a load of 2.5 kg (5 lb).  However, flexible metal tapes of 10 m (25 ft) or less that are not normally used under tension shall be tested with no tension applied.  All tapes shall be supported throughout on a horizontal flat surface whenever tested.

(Amended 1972)

| Table 2. Maintenance and Acceptance Tolerances, in Excess and in Deficiency, for Metal Tapes |  |
| --- | --- |
| Nominal Interval from Zero | Tolerance |
| feet | inch |
| 6 or less 7 to 30, inclusive 31 to 55, inclusive 56 to 80, inclusive 81 to 100, inclusive | 1/32 1/16 1/8 3/16 ¼ |